# Cadence Layout for SRAM Phase I

Bill Hung, Dennis Wang

Table of Contents

Cadence Layout for SRAM Phase I	1

Opening a New Layout and New Library	2

SRAM Cell Transistors Layout	5

Drawing NMOS	5

Exploring Cadence Transistor Sizes	7

NMOS	7

PMOS	9

Testing NMOS and PMOS Extraction	11

NMOS	11

PMOS	12

Half SRAM Cell Transistors Layout	13

Wiring within the SRAM Cell with Contacts	17

Contacts	18

Wiring word line	19

Wiring BL,−BL, Q−,Q and Vdd	20

Instantiate N-tap	21

Minimizing Wires	21

Connecting the Grounds	22

Connecting the VDD	24

Final Layout	25

Testing with Repetitive Pattern	26

APPENDIX	27

Cadence Shortcut Keys	27

Failure Notes, mistakes that I learned from trial and error	27

HSPICE Minimum NMOS Extracted Netlist	28

HSPICE Minimum PMOS Extracted Netlist	29

HSPICE Q-bar between M1 NMOS drain and M2 PMOS drain	30

HSPICE Wiring BL,−BL, Q−,Q and Vdd	31

HSPICE After Ground Metal 2 Rail	32

HSPICE After Ground Metal 3 Connection	33

Final HSPICE SRAM Cell Extracted	34

List of Tables

Table 1 PMOS Dimensions in Library NCSU_TechLib_tsmc03d Layout	5

Table 2 Minimum PMOS and NMOS Sizes from Library NCSU_TechLib_tsmc03d	5

Table 3 Actual NMOS Size for Phrase I	7

Table 4 Actual PMOS Size for Phrase I	9

Table 5 Cadence Shortcut Keys	27

### Opening a New Layout and New Library

File->New->Library

Put “sram” in Library field in Library Manager and press Enter.

Make the folder and proceed.

OK

OK

OK

## SRAM Cell Transistors Layout

Ultimately, want to draw the following layout.

### Drawing NMOS

For now. Open NCSU_TechLib_tsmc03d->nmos->layout, and try to draw an NMOS

myself.

Table 1 PMOS Dimensions in Library NCSU_TechLib_tsmc03d Layout

| Name | Height x Width |
| --- | --- |
| Contact (cc) | 24 x 24 |
| Poly (poly) | 1.6 x 1.0 |
| Drain Metal (metal1) | 1.0 x 0.48 |
| Well (nactive) | 1.0 x 2.2 |
| Circumference (nselect) | 1.6 x 2.68 |

Table 2 Minimum PMOS and NMOS Sizes from Library NCSU_TechLib_tsmc03d

| Name | Width / Length |
| --- | --- |
| NMOS | 0.36um / 0.24um |
| PMOS | 0.36um / 0.24um |

The layout by opening the NCSU_TechLib_tsmc03d library.

“r” nselect, nactive. Everything is dg, not nt in the LSW window.

“r” metal1, cc. Notice the contacts have to be exactly 0.24um x 0.24um or DRC error.

“r” poly

“q”, then change the left, right, bottom, top to change the size.

### Exploring Cadence Transistor Sizes

#### NMOS

Table 3 Actual NMOS Size for Phrase I

| Name | Width x Height |
| --- | --- |
| nselect (same as library) | 1.92 x 1.08 |
| nactive (Well) | 1.44 x 0.48 |
| metal1 (Drain metal) | 0.48 x 0.48 |
| poly | 0.24 x 0.96 |
| cc (contacts) | 0.24 x 0.24 |

nactive below, library nmos above.

Metal1

Poly

Nmos from library (left), and my nmos (right). They are exactly the same. DRC error free, extract error free.

#### PMOS

The library cell by opening the NCSU_TechLib_tsmc03d library

Table 4 Actual PMOS Size for Phrase I

| Name | Width x Height |
| --- | --- |
| Nwell | 2.880 x 1.920 |
| pselect (orange box) | 1.920 x 1.080 |
| Pactive | 1.44 x 0.48 |
| metal1 | 0.48 x 0.48 |
| cc (contacts) | 0.24 x 0.24 |
| Poly | 0.24 x 0.96 |

nwell and pselect (orange box)

pactive, metal1

cc (contacts), poly

DRC->   Total errors found: 0

Extract ->  Total errors found: 0

### Testing NMOS and PMOS Extraction

#### NMOS

Verify->Extract

Total errors found: 0

Open to the 3mos->extract

Tools->Analog Environment->Setup Environment

Check “hspice” and “flat”, OK

Simulation->Netlist->Create Final

Save As -> ~/141/phrase1/min_nmos.sp

The extracted width and length agree with the layout (360nm x 240nm). The extracted hspice netlist is attached at the end.

#### PMOS

Open to the 3mos->extract

Tools->Analog Environment->Setup Environment

Check “hspice” and “flat”, OK

Simulation->Netlist->Create Final

Save As -> ~/141/phrase1/min_pmos.sp

The extracted width and length agree with the layout (360nm x 240nm). The extracted hspice netlist is attached at the end.

### Half SRAM Cell Transistors Layout

This is the goal of the layout [Nikolic, lec 17 page 7]

Layout Example Posted by TA [EE 141, Spring 2006]

Stick Diagram of the SRAM Cell Layout

BL

WL

M5

Q

M1

GND

VDD

Q

M2

Q

M4

M3

M6

Q

VDD

Q

Q

GND

WL

BL

Figure 1SRAM Cell Layout Stick Diagram

Edit->Other->Rotate to rotate the transistors. cp and align them to meet the schematic given for 65nm SRAM

Min distance (3.5 lambda) between nmos and pmos with DRC error.

Access transistor (top), pull down nmos (left bottom), and pull up pmos (right)

M1_POLY to metal1 min distance (3 lambda)

M1_POLY contact and poly added to pmos

Active contact to active spacing (5 lambda)

Poly to poly contact (4 lambda), active contact to active spacing (3 lambda), so this makes the 2 pmos has 6 lambda apart instead of 5 lambda apart.

## Wiring within the SRAM Cell with Contacts

Q are connected with metal 1 between M1 NMOS drain and M2 PMOS drain. DRC error free, extract was error free. The Hspice attached in appendix. The problem here is the metal 1 between M1 and M2 using metal 1 will connect the gate of pmos M4 also, which is not correct.

#### Contacts

Contacts M1_POLY(left), M1_P(middle), M1_N (right)

(from left) Contact M5_M4, M4_M3, M3_M2, M2_M1, NTAP

(from left) NTAP = nwell/nselect/cc/m1 or nwell/m1_n, ntap, M1_P, PTAP

Now I try to make contacts on the gate of M5

M2_M1 contact (left), M3_M2 contact (right) placed at the same place, onto of gate of M5 for Word Line (WL) connections.

### Wiring word line

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/SRAM_CELL/HSPICES/

* EXTRACTED/NETLIST/SRAM_CELL.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 17 23:45:35 2006

M0 5 7 14 2 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 12 4 6 2 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 13 7 9 11 TSMC25DN L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 10 4 15 11 TSMC25DN L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 8 3 13 11 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 15 3 1 11 TSMC25DN L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

Using M3, the horizontal wire in the graph, to connect the gate of the access transistors since according to the Layout Example Posted by TA, if we use M3 for vertical wiring, we need to use a lot of contact M1_M2 and M2_M3.

### Wiring BL,−BL, Q−,Q and Vdd

There is a problem when we use M1_M2 as a connection to verticalBL−. The minimum spacing between M1 is 0.36um (3 lambda), and The minimum spacing between M2 is 0.48um (4 lambda).

Therefore, I move M1_M2 contact on the left (WL connection) down, and move M1_M2 contact on the right (BL−connection) away. Now it is DRC free again. Using M2 vertically to wire Vdd between the 2 pmos. The result is shown below. Hspice Netlist attached at the end.

### Instantiate N-tap

First attempt: laying down the N-tap in the middle and as close as to center, but unfortunately, it didn’t work

Second attempt: putting the N-tap in the bottom-right corner, and it works

### Minimizing Wires

Wire minimum width 0.36um, 3 lambda

### Connecting the Grounds

The M1_M2 contact needs 4 lambda from metal 2 and 3 lambda from metal 1. So I moved the metal 2 of the bitline-bar to the right for half a lambda. Then DRC free.

The metal 1 between M1_M2 contact and metal2 sticking up from M1_M2 contact were drawn. The spacing between metal 2 is at least 4 lambda. So move the gound metal 2 to the right. Then DRC free.

Move the metal 2 contact to the right to remove metal 2 DRC error. They need 4 lambda apart.

After long discussion with Dennis, we decided in order to put the ntap in the cell, we have no choice but to increase the horizontal distance by half a lambda. The result is DRC free.

Connecting the grounds through metal1-metal2-metal3-metal2-metal1. DRC error free, extract error free.

### Connecting the VDD

The ground is connected to the ntap through metal 1. The wire between them has to be 4 lambda instead of the minimum 3 lambda or DRC error.

## Final Layout

After putting the ptaps at the corners. DRC and extra error free. The hspice gives the right circuit (attached). YES!

Final Postscript Plot

### Testing with Repetitive Pattern

The layout we have so far can not be flipped correctly since the gates of the access transistor should be center at the rotation axis.

After move the center of the poly contact, it is now DRC free

APPENDIX

## Cadence Shortcut Keys

Table 5 Cadence Shortcut Keys

| Shortcut Key | Meaning |
| --- | --- |
| C | Copy |
| M -> Flip Upside Down | Flip |
| O | Contact |

## Failure Notes, mistakes that I learned from trial and error

- contacts have to be exactly 0.24um x 0.24um or DRC error.

- Tried the exact dimension in Table 1, they didn’t work for me. Those are the sizing parameters from the library layout. I made an nmos with the same exact dimension of the library transistor, but it ends up with lots of DRC errors. So finally opening the inverter from HW instead for sizing.

## HSPICE Minimum NMOS Extracted Netlist

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/3MOS/HSPICES/

* EXTRACTED/NETLIST/3MOS.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 16 23:33:20 2006

M0 5 3 6 2 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 7 4 1 2 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

## HSPICE Minimum PMOS Extracted Netlist

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/3MOS/HSPICES/

* EXTRACTED/NETLIST/3MOS.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 17 00:39:30 2006

M0 11 2 8 5 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 6 13 12 5 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 9 4 10 3 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M3 14 7 1 3 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

## HSPICE Q-bar between M1 NMOS drain and M2 PMOS drain

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/SRAM_CELL/HSPICES/

* EXTRACTED/NETLIST/SRAM_CELL.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 17 20:38:00 2006

M0 10 12 9 5 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 4 10 12 5 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 10 12 7 2 TSMC25DN L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 8 10 12 2 TSMC25DN L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 6 3 10 2 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 12 11 1 2 TSMC25DN L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

The node 12 is Q-bar. I noticed the pmos M1 in the netlist and the two big nmos (M2 and M3 in the netlist) are connected.  This is good.

The netlist suggests that with a metal 1 connecting M1 and M2 in the stick diagram (Figure 1) will connect the gate of M4. This is what we want

## HSPICE Wiring BL,−BL, Q−,Q and Vdd

In the netlist extraction (we can’t tell if BL will be properly connected at this stage, but we are sure that the two half cells are connected together; Qbar corresponding to node 9 whereas Q corresponding to node 10. Vdd is node 4)

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/SRAM_CELL/HSPICES/

* EXTRACTED/NETLIST/SRAM_CELL.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 18 12:19:53 2006

M0 9 10 4 3 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 4 9 10 3 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 9 10 6 2 TSMC25DN L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 7 9 10 2 TSMC25DN L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 5 8 9 2 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 10 8 1 2 TSMC25DN L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

## HSPICE After Ground Metal 2 Rail

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/SRAM_CELL/HSPICES/

* EXTRACTED/NETLIST/SRAM_CELL.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 18 17:54:03 2006

M0 9 10 4 3 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 4 9 10 3 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 9 10 6 2 TSMC25DN L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 7 9 10 2 TSMC25DN L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 5 8 9 2 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 10 8 1 2 TSMC25DN L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

## HSPICE After Ground Metal 3 Connection

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/SRAM_CELL/HSPICES/

* EXTRACTED/NETLIST/SRAM_CELL.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 18 20:57:07 2006

M0 3 8 4 1 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 4 3 8 1 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 3 8 7 5 TSMC25DN L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 7 3 8 5 TSMC25DN L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 6 2 3 5 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 8 2 9 5 TSMC25DN L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END

## Final HSPICE SRAM Cell Extracted

* # FILE NAME: /HOME/AA/UGRAD/BILLHUNG/CADENCE/SIMULATION/SRAM_CELL/HSPICES/

* EXTRACTED/NETLIST/SRAM_CELL.C.RAW

* NETLIST OUTPUT FOR HSPICES.

* GENERATED ON MAR 18 22:14:53 2006

M0 2 5 1 1 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 1 2 5 1 TSMC25DP L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 2 5 7 7 TSMC25DN L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 7 2 5 7 TSMC25DN L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 4 3 2 7 TSMC25DN L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 5 3 6 7 TSMC25DN L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

* INCLUDE FILES

* END OF NETLIST

.TEMP    25.0000

.OP

.save

.OPTION  INGOLD=2 ARTIST=2 PSF=2

+        PROBE=0

.END