- The Term Project of UC Berkeley CS 150
Components and Design Techniques for Digital Systems
Audio Broadcasting Over Ethernet
Filtering Network Data into Audio Data

Eth2Data.v

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| DIn | 32 | I | Packet data input bus |
| InValid | 1 | I | Indicates that DIn is valid on this cycle |
| InPacketValid | 1 | I | Indicates the end of a packet with a valid CRC |
| InPacketInvalid | 1 | I | Indicates the end of a packet with an invalid CRC |
| EthernetClock | 1 | I | 25MHz Ethernet Clock |
| EthernetReset | 1 | I | EthernetClock synchronous reset |
| OutClock | 1 | I | 12.288MHz Audio Clock |
| OutReset | 1 | I | OutClock synchronous reset |
| DOut | 32 | O | Audio data output bus |
| OutRequest | 1 | I | Request for a new audio word output |
| OutValid | 1 | O | Indicates that DOut is currently valid |
| datawords |  | P | There should be a datawords parameter, which will allow you to easily change the size of the packet decoded by this module |

Audio Volume Control

SHAPE  \* MERGEFORMAT

AC97Controller.v

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| AP_SDATA_OUT | 1 | O | AC97 serial data output to the LM4549A |
| AP_BIT_CLOCK | 1 | I | A 12.288MHz gated clock. You should use this for all audio circuitry. |
| AP_SDATA_IN | 1 | I | AC97 serial data input from the LM4549A |
| AP_SYNC | 1 | O | AC97 frame synchronization signal |
| AP_RESET_ | 1 | O | Active low (Notice the _) reset for the LM4549A. This will gate the AP_BIT_CLOCK |
| AP_PC_BEEP | 1 | O | PC beep output. Tie to 1’b0. |
| Reset | 1 | I | System reset, synchronous to Clock, used with LocalResetGen |
| Clock | 1 | I | System 27MHz free running clock, used with LocalResetGen |
| PCM_DIn | 32 | I | 32bit PCM data input. Bits [31:16] is the left sample, bits [15:0] is the right sample. |
| PCM_InValid | 1 | I | Data valid input. Indicates that PCM_DIn is valid. Driven by Eth2Data. |
| PCM_InRequest | 1 | O | PCM_DIn request. Causes a read from the Eth2Data FIFO. |
| PCM_DOut | 32 | O | PCM data output. This would carry the data read from the microphone. Unused this semester. |
| PCM_OutValid | 1 | O | Indicates that PCM_DOut has valid data. Unused this semester. |
| CMD_AIn | 7 | I | The address of the AC97 register to write. |
| CMD_DIn | 16 | I | Data to write to the AC97 register indicated by CMD_AIn. |
| CMD_InValid | 1 | I | Indicates that CMD_DIn and CMD_AIn are valid in response to CMD_InRequest. |
| CMD_InRequest | 1 | O | Request by this controller for the next AC97 register to write. |
| AudioReset | 1 | O | Audio system reset. Generated by LocalResetGen. |
| AudioClock | 1 | O | 12.288MHz audio system clock. Another name for AP_BIT_CLOCK. |

FullVolumeControl.v

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| VolMaster | 6 | I | Volume control for all audio being played out the headphone jack on the CaLinx2 board. Notice, there is a blue potentiometer which affects this also. |
| VolLineOut | 6 | I | Volume control for all audio being played out the RCA Line Out Jacks on the CaLinx2 board. |
| VolLineIn | 6 | I | Unused this semester. |
| VolWave | 6 | I | Volume control for all PCM data from the FPGA. |
| VolMic | 6 | I | Unused this semester |
| VolRecordGain | 6 | I | Unused this semester |
| RecordSource | 3 | I | Unused this semester |
| CMD_AOut | 7 | O | The address of the AC97 register to write. |
| CMD_DOut | 16 | O | Data to write to the AC97 register indicated by CMD_AOut. |
| CMD_OutValid | 1 | O | Indicates that CMD_DOut and CMD_AOut are valid in response to CMD_OutRequest. |
| CMD_OutRequest | 1 | I | Request from AC97Controller for the next AC97 register to write. |
| AudioReset | 1 | I | Audio system reset. |
| AudioClock | 1 | I | 12.288MHz audio system clock. Another name for AP_BIT_CLOCK. |