EECS 141: Spring 2006                                                           Bill Hung and Dennis Wang


Dynamic Logic

Table of contents

2Finding Sizing for 5-stage Dynamic Logic


2Matlab Code


3Matlab Result


4Sweeping Transistor – NAND Gate Test


4HSPICE Code


4Awave Result


5Sweeping Transistor – NAND Gate with subcircuit


5Sweep_transistor.sp


5Sweep_transistor.sp


6Gate.sp


7Awave Result


8Paramters


8Sweeping Static Logic Transistors


8Sweep Static Logic HSPICE


10Awave Code


11Dynamic Logic Gates


125-Stage Dynamic Logic that Does Not Work


12HSPICE


13Awaves


146-Stage Dynamic Logic


14HSPICE


16Awaves




Finding Sizing for 5-stage Dynamic Logic
Dynamic Logic chain with 3fF-not-nand-not-nand-not-cload. Try to find the sizing for HSPICE simulation.
Matlab Code
% Dynamic Logic

% 3fF-NOT-NAND-NOT-NAND-NOT-Cload

%Parameters

g1=1; g2=5/3; g3=1; g4=5/3; g5=1;
b1=1; b2=4; b3=1; b4=8; b5=1;
nstage=5;
%find cload

cw=5.5; %from HSPICE extraction 

cox=6; co=0.31; w=0.36; l=0.24;
cg = cox*w*l + 2*co*w;
cload = cw + 2*cg*32
%Find h

G=g1*g2*g3*g4*g5; B=b1*b2*b3*b4*b5; F=(cload)/(3*cg);
H=G*B*F
h=H^(1/nstage)
%Find f

f(1)=h/(b1*g1);
f(2)=h/(b2*g2);
f(3)=h/(b3*g3);
f(4)=h/(b4*g4);
f(5)=h/(b5*g5);
s(5)=(cload)/f(5);  s(4)=s(5)/f(4);
s(3)=s(4)/f(3);  s(2)=s(3)/f(2);
s(1)=s(2)/f(1)
% add cw1

g1=1; g2=1; g3=5/3; g4=1;
b1=1; b2=4; b3=1; b4=1;
nstage2=4;
cw1=1.8641; %from HSPICE

cload = s(5)*8+cw1;
F2 = (cload)/(3*cg);
    %cw1 is M4 of the predecoder = 1.8641

B2= b1*b2*b3*b4;
G2 = g1*g2*g3*g4;
H2 = F2* G2* B2
h2 = H2^(1/nstage2)
f1(1)=h2/(b1*g1);
f1(2)=h2/(b2*g2);
f1(3)=h2/(b3*g3);
f1(4)=h2/(b4*g4);
s1(4)=(cload)/f1(4);
s1(3)=s1(4)/f1(3);
s1(2)=s1(3)/f1(2);
s1(1)=s1(2)/f1(1);
%get w

cox=6; co=0.31; w=0.36; l=0.24;
cg = cox*w*l + 2*co*w;
inv_min = 3*cg;
nand3_min = 5*cg;
w(5)=s(5)/nand3_min;
w(4)=s1(4)/inv_min;
w(3)=s1(3)/nand3_min;
w(2)=s1(2)/inv_min;
w(1)=s1(1)/inv_min
Matlab Result
cload =   52.9624

H =  2.1160e+003

h =    4.6249

s =  Columns 1 through 2 

    2.2248   10.2895

  Columns 3 through 4 

    7.1383   33.0139

  Column 5 

   11.4515

H2 =  280.1037

h2 =

    4.0910
w =

  Columns 1 through 2 

    1.0000    4.0910

  Columns 3 through 4 

    2.5104   10.2702

  Column 5 
    3.0883
Sweeping Transistor – NAND Gate Test

Trying to sweep logic gate sizes in HSPICE.

HSPICE Code

one transistor sweep

.lib '/home/ff/ee141/MODELS/g25.mod' TT

*****************************************

* Parameter   

*****************************************

.param vddp = 2.5

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.param multiplier = 2.0449

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

.subckt NAND3 Vdd Gnd VinA VinB VinC Vout

Mp1 Vout VinA Vdd Vdd pmos l=0.24u w='wpmin'

Mp2 Vout VinB Vdd Vdd pmos l=0.24u w='wpmin'

Mp3 Vout VinC Vdd Vdd pmos l=0.24u w='wpmin'

Mn1 Vmid1 VinA Gnd Gnd nmos l=0.24u w='wnmin*3'

Mn2 Vmid2 VinB Vmid1 Gnd nmos l=0.24u w='wnmin*3'

Mn3 Vout VinC Vmid2 Gnd nmos l=0.24u w='wnmin*3'

.ends

xnand vdd 0 vin1 vdd vdd vout  NAND3 M=multiplier

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

[image: image8.png]‘AvanWaves ¥-2004.03 (20040116)

Design Panels Window Measure Configuration Tools

~=loix|
Help

FIEE

] 2 vl 2l 2l el 2]

Voltages (in)

one transistor sweep

540 5.6n
Time (in) (TIME)




.tran 0.05ns 10ns sweep multiplier 1 5 1

.end
Awave Result

Sweeping Transistor – NAND Gate with subcircuit

Sweep one NAND gate with subcircuit in another file.
Sweep_transistor.sp

one transistor sweep

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc 'gate.sp'

*****************************************

* Parameter   

*****************************************

.param vddp = 2.5

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.param multiplier = 2.0449

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

xnand vdd 0 vin1 vdd vdd vout  NAND3 M='multiplier'

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.05ns 10ns sweep multiplier 1 5 1

.end

Sweep_transistor.sp

one transistor sweep

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc 'gate.sp'

*****************************************

* Parameter   

*****************************************

.param vddp = 2.5

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.param multiplier = 2.0449

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

xnand vdd 0 vin1 vdd vdd vout  NAND3 M='multiplier'

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.05ns 10ns sweep multiplier 1 5 1

.end

Gate.sp

.param wnmin='0.36u'

.param wpmin='2*wnmin'

.subckt INV vin vout vdd vss

M1 vout vin vss vss  nmos l=0.24u w='wnmin'

M2 vout vin vdd vdd  pmos l=0.24u w='wpmin'

.ends

.subckt NAND3 Vdd Gnd VinA VinB VinC Vout

Mp1 Vout VinA Vdd Vdd pmos l=0.24u w='wpmin'

Mp2 Vout VinB Vdd Vdd pmos l=0.24u w='wpmin'

Mp3 Vout VinC Vdd Vdd pmos l=0.24u w='wpmin'

Mn1 Vmid1 VinA Gnd Gnd nmos l=0.24u w='wnmin*3'

Mn2 Vmid2 VinB Vmid1 Gnd nmos l=0.24u w='wnmin*3'

Mn3 Vout VinC Vmid2 Gnd nmos l=0.24u w='wnmin*3'

.ends

.subckt Sram Vdd Gnd vword vbit vbitbar

    *M<name> <drain> <gate> <source> <bulk> <model> <geometry>

M0 2 5 Vdd Vdd pmos L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M1 Vdd 2 5 Vdd pmos L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=273.599990319867E-15 PD=1.7999999499807E-6 PS=1.7999999499807E-6 M=1

M2 2 5 0 0 nmos L=240E-9 W=480E-9 AD=165.599996280845E-15

+AS=288.000011209114E-15 PD=779.999993483216E-9 PS=1.67999996847357E-6 M=1

M3 0 2 5 0 nmos L=240E-9 W=480E-9 AD=288.000011209114E-15

+AS=165.599996280845E-15 PD=1.67999996847357E-6 PS=779.999993483216E-9 M=1

M4 vbit vword 2 0 nmos L=240E-9 W=360E-9 AD=273.599990319867E-15

+AS=165.599996280845E-15 PD=1.7999999499807E-6 PS=779.999993483216E-9 M=1

M5 5 vword vbitbar 0 nmos L=240E-9 W=360E-9 AD=165.599996280845E-15

+AS=273.599990319867E-15 PD=779.999993483216E-9 PS=1.7999999499807E-6 M=1

.ends

.subckt NOR Vdd Gnd VinA VinB Vout

Mp1 Vout VinA Vdd Vdd pmos l=0.24u w=0.6u

Mp2 Vout VinB Vdd Vdd pmos l=0.24u w=0.6u

Mn1 Vout VinA vmid Gnd nmos l=0.24u w=0.72u

Mn2 vmid VinB Gnd Gnd nmos l=0.24u w=0.72u

[image: image9.png]Help

o o T o B
= =
2| ¢l &l £l ¥l ®]

one transistor sweep

| feo] &)

Voltages (in)

n

&
Time (in) (TIME)




.ends

Awave Result

Paramters

Cw = 12.60438 fF
Cw1 = 1.8641f
SRAM pitch = 1.92um
PMOS-NMOS ratio: 1.6 (Rabaey, 209)
Sweeping Static Logic Transistors 

The goal is the find the optimal delay by sweeping the width of transistors used in the decoder circuit.
Sweep Static Logic HSPICE

PHASE II DYNAMIC LOGIC

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc  'logic.sp'

*****************************************

* Parameter   

*****************************************

.param vddp = 2.5

.param multi_nand1 = 4.09

.param multi_inv2 = 2.51

.param multi_nand3 = 10.27

.param multi_inv3 = 3.0

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

* params = vlow vhigh delay rise fall pulse_width period

* example VIN IN GND PULSE 0 5 .5n .1n .1n .5n 2n

Vin vin1 0 pulse(0v 2.5v 0.1n 50ps 50ps 5ns 10ns)

*****************************************

* Netlist

*****************************************

xinv1 vin1 vin2 vdd 0 INV

xnand1_b1 vdd 0 vdd vdd vin2 vin3 NAND3 M='multi_nand1'

xnand1_b2 vdd 0 0 0 vin2 x NAND3 M='multi_nand1'

xnand1_b3 vdd 0 0 0 vin2 y NAND3 M='multi_nand1'

xnand1_b4 vdd 0 0 0 vin2 z NAND3 M='multi_nand1'

xinv2 vin3 vin4 vdd 0 INV M='multi_inv2'

cw1 vin5 0 1.8641f

xnand2_b1 vdd 0 vdd vdd vin4 vin5 NAND3 M='multi_nand3'

xnand2_b2 vdd 0 0 0 vin5 a NAND3 M='multi_nand3'

xnand2_b3 vdd 0 0 0 vin5 b NAND3 M='multi_nand3'

xnand2_b4 vdd 0 0 0 vin5 c NAND3 M='multi_nand3'

xnand2_b5 vdd 0 0 0 vin5 d NAND3 M='multi_nand3'

xnand2_b6 vdd 0 0 0 vin5 e NAND3 M='multi_nand3'

xnand2_b7 vdd 0 0 0 vin5 f NAND3 M='multi_nand3'

xnand2_b8 vdd 0 0 0 vin5 g NAND3 M='multi_nand3'

xinv3 vin5 vout vdd 0 INV M='multi_inv3'

cload vout 0 12.60438fF

* SRAM Chain

xsram0 vdd 0 vout vbit0 vbitbar0 Sram

xsram1 vdd 0 vout vbit1 vbitbar1 Sram

xsram2 vdd 0 vout vbit2 vbitbar2 Sram

xsram3 vdd 0 vout vbit3 vbitbar3 Sram

xsram4 vdd 0 vout vbit4 vbitbar4 Sram

xsram5 vdd 0 vout vbit5 vbitbar5 Sram

xsram6 vdd 0 vout vbit6 vbitbar6 Sram

xsram7 vdd 0 vout vbit7 vbitbar7 Sram

xsram8 vdd 0 vout vbit8 vbitbar8 Sram

xsram9 vdd 0 vout vbit9 vbitbar9 Sram

xsram10 vdd 0 vout vbit10 vbitbar10 Sram

xsram11 vdd 0 vout vbit11 vbitbar11 Sram

xsram12 vdd 0 vout vbit12 vbitbar12 Sram

xsram13 vdd 0 vout vbit13 vbitbar13 Sram

xsram14 vdd 0 vout vbit14 vbitbar14 Sram       

xsram15 vdd 0 vout vbit15 vbitbar15 Sram       

xsram16 vdd 0 vout vbit16 vbitbar16 Sram    

xsram17 vdd 0 vout vbit17 vbitbar17 Sram                                    

xsram18 vdd 0 vout vbit18 vbitbar18 Sram

xsram19 vdd 0 vout vbit19 vbitbar19 Sram

xsram20 vdd 0 vout vbit20 vbitbar20 Sram

xsram21 vdd 0 vout vbit21 vbitbar21 Sram

xsram22 vdd 0 vout vbit22 vbitbar22 Sram

xsram23 vdd 0 vout vbit23 vbitbar23 Sram

xsram24 vdd 0 vout vbit24 vbitbar24 Sram

xsram25 vdd 0 vout vbit25 vbitbar25 Sram

xsram26 vdd 0 vout vbit26 vbitbar26 Sram       

xsram27 vdd 0 vout vbit27 vbitbar27 Sram       

xsram28 vdd 0 vout vbit28 vbitbar28 Sram    

xsram29 vdd 0 vout vbit29 vbitbar29 Sram                                    

xsram30 vdd 0 vout vbit30 vbitbar30 Sram

xsram31 vdd 0 vout vbit31 vbitbar31 Sram

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.01ns 11ns sweep multi_nand3 9.27 11.27 1 sweep multi_nand1 3.09 5.09 1 sweep  multi_inv3 3.09 5.09 1 

.meas tran tplh trig V(vin1) val='(2.5*.5)' rise=1 targ V(vout)

+ val='(2.5*.5)' rise=1

.end

Awave Code
The blue line is the static decoder. Not swept.
[image: image1.png]04.03 (20040116)
Design Panels Window Measure Configuration Tools

~=loix|
Help

FIEE

2] 2l ¥l 8] el el 2|

Voltages (in)

phase i dynaic logic

6 7n
Time (in) (TIME)

n





The multiplier swept.

[image: image2.png]‘AvanWaves ¥-2004.03 (20040116) -1o) x|

Design Panels Window Measure Configuration Tools Help

T fos v v
A& 10 &) 2| ¥el %l ¢l ¥el 2|
25
ID1:tr0:v (v(vout))
BD0:tr0:v (vout)) 2
| p1:tr0:v (vouty) =
Tsoezr | S s
81,9.27 g
s et L
83,1.13e+1 >
412300 soim
.
o
Time (lin) (TIME) on





Dynamic Logic Gates
[image: image3.emf]
2-input-NAND with inverter [Nikolic, Lecture 19, page 12]

3fF-NOT-NAND-NOT-NAND-NOT-Cload dynamic circuit
 SHAPE  \* MERGEFORMAT 



5-Stage Dynamic Logic that Does Not Work

The problem with 5-stages is when all inputs are on, the output oscillate between vdd and zero. This is not suitable for a decoder.

HSPICE

PHASE II DYNAMIC LOGIC

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc  'gate.sp'

*****************************************

* Parameter   

*****************************************

.param vddp = 2.5

.param multi_nand1 = 4.09

.param multi_inv2 = 2.51

.param multi_nand3 = 10.27

.param multi_inv3 = 3.0

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

* params = vlow vhigh delay rise fall pulse_width period

* example VIN IN GND PULSE 0 5 .5n .1n .1n .5n 2n

Vin vin1 0 pulse(0v 2.5v 0.1n 0.1n 50ps 5ns 10ns)

Vclk clk 0 pulse(0v 2.5v 0.1n 0.1n 50ps 2.5ns 5ns)

*****************************************

* Netlist

*****************************************

xinv1 vin1 vin2 vdd 0 INV

xnand1_b1 vdd 0 vin2 vin2 vin2 vin3 clk vin4 DNAND3 M='multi_nand1'

xnand1_b2 vdd 0 0 0 vin2 x clk xx  DNAND3 M='multi_nand1'

xnand1_b3 vdd 0 0 0 vin2 y clk yy DNAND3 M='multi_nand1'

xnand1_b4 vdd 0 0 0 vin2 z clk zz  DNAND3 M='multi_nand1'

xinv2 vin3 vin4 vdd 0 INV M='multi_inv2'

cw1 vin5 0 1.8641f

xnand2_b1 vdd 0 vin4 vin4 vin4 vin5 clk vout DNAND3 M='multi_nand3'

xnand2_b2 vdd 0 0 0 vin5 a clk aa DNAND3 M='multi_nand3'

xnand2_b3 vdd 0 0 0 vin5 b clk bb DNAND3 M='multi_nand3'

xnand2_b4 vdd 0 0 0 vin5 c clk cc DNAND3 M='multi_nand3'

xnand2_b5 vdd 0 0 0 vin5 d clk dd DNAND3 M='multi_nand3'

xnand2_b6 vdd 0 0 0 vin5 e clk ee DNAND3 M='multi_nand3'

xnand2_b7 vdd 0 0 0 vin5 f clk ff DNAND3 M='multi_nand3'

xnand2_b8 vdd 0 0 0 vin5 g clk gg DNAND3 M='multi_nand3'

xinv3 vin5 vout vdd 0 INV M='multi_inv3'

cload vout 0 12.60438fF

* SRAM Chain

xsram0 vdd 0 vout vbit0 vbitbar0 Sram

xsram1 vdd 0 vout vbit1 vbitbar1 Sram

xsram2 vdd 0 vout vbit2 vbitbar2 Sram

xsram3 vdd 0 vout vbit3 vbitbar3 Sram

xsram4 vdd 0 vout vbit4 vbitbar4 Sram

xsram5 vdd 0 vout vbit5 vbitbar5 Sram

xsram6 vdd 0 vout vbit6 vbitbar6 Sram

xsram7 vdd 0 vout vbit7 vbitbar7 Sram

xsram8 vdd 0 vout vbit8 vbitbar8 Sram

xsram9 vdd 0 vout vbit9 vbitbar9 Sram

xsram10 vdd 0 vout vbit10 vbitbar10 Sram

xsram11 vdd 0 vout vbit11 vbitbar11 Sram

xsram12 vdd 0 vout vbit12 vbitbar12 Sram

xsram13 vdd 0 vout vbit13 vbitbar13 Sram

xsram14 vdd 0 vout vbit14 vbitbar14 Sram       

xsram15 vdd 0 vout vbit15 vbitbar15 Sram       

xsram16 vdd 0 vout vbit16 vbitbar16 Sram    

xsram17 vdd 0 vout vbit17 vbitbar17 Sram                                    

xsram18 vdd 0 vout vbit18 vbitbar18 Sram

xsram19 vdd 0 vout vbit19 vbitbar19 Sram

xsram20 vdd 0 vout vbit20 vbitbar20 Sram

xsram21 vdd 0 vout vbit21 vbitbar21 Sram

xsram22 vdd 0 vout vbit22 vbitbar22 Sram

xsram23 vdd 0 vout vbit23 vbitbar23 Sram

xsram24 vdd 0 vout vbit24 vbitbar24 Sram

xsram25 vdd 0 vout vbit25 vbitbar25 Sram

xsram26 vdd 0 vout vbit26 vbitbar26 Sram       

xsram27 vdd 0 vout vbit27 vbitbar27 Sram       

xsram28 vdd 0 vout vbit28 vbitbar28 Sram    

xsram29 vdd 0 vout vbit29 vbitbar29 Sram                                    

xsram30 vdd 0 vout vbit30 vbitbar30 Sram

xsram31 vdd 0 vout vbit31 vbitbar31 Sram

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.1ns 16ns *sweep multi_nand3 

.meas tran tplh trig V(vin1) val='(2.5*.5)' rise=1 targ V(vout)

+ val='(2.5*.5)' rise=1

.end

Awaves
  SHAPE  \* MERGEFORMAT 



6-Stage Dynamic Logic

6 Stage Dynamic Logic will have the output oscillating during a read. However, for a decoder design, we need to have the output stay high for multiple cycles.
HSPICE

PHASE II DYNAMIC LOGIC

.lib '/home/ff/ee141/MODELS/g25.mod' TT

.inc  'gate.sp'

*****************************************

* Parameter   

*****************************************

.param vddp = 2.5

.param multi_nand1 = 4.09

.param multi_inv2 = 2.51

.param multi_nand3 = 10.27

.param multi_inv3 = 3.0

.param multi_inv4 = 4.6

*****************************************

* Sources

*****************************************

Vdd vdd 0 'vddp'

* params = vlow vhigh delay rise fall pulse_width period

* example VIN IN GND PULSE 0 5 .5n .1n .1n .5n 2n

Vin vin1 0 pulse(0v 2.5v 0.1n 0.1n 50ps 5ns 10ns)

Vclk clk 0 pulse(0v 2.5v 0.1n 0.1n 50ps 2.5ns 5ns)

*****************************************

* Netlist

*****************************************

xinv1 vin1 vin2 vdd 0 INV

xinv2 vin2 vin3 vdd 0 INV M='multi_inv2'

xnand1_b1 vdd 0 vin3 vin3 vin3 vin4 clk vin5 DNAND3 M='multi_nand1'

xnand1_b2 vdd 0 0 0 vin3 x clk xx  DNAND3 M='multi_nand1'

xnand1_b3 vdd 0 0 0 vin3 y clk yy DNAND3 M='multi_nand1'

xnand1_b4 vdd 0 0 0 vin3 z clk zz  DNAND3 M='multi_nand1'

xinv3 vin4 vin5 vdd 0 INV M='multi_inv3'

cw1 vin5 0 1.8641f

xnand2_b1 vdd 0 vin5 vin5 vin5 vin6 clk vout DNAND3 M='multi_nand3'

xnand2_b2 vdd 0 0 0 vin5 a clk aa DNAND3 M='multi_nand3'

xnand2_b3 vdd 0 0 0 vin5 b clk bb DNAND3 M='multi_nand3'

xnand2_b4 vdd 0 0 0 vin5 c clk cc DNAND3 M='multi_nand3'

xnand2_b5 vdd 0 0 0 vin5 d clk dd DNAND3 M='multi_nand3'

xnand2_b6 vdd 0 0 0 vin5 e clk ee DNAND3 M='multi_nand3'

xnand2_b7 vdd 0 0 0 vin5 f clk ff DNAND3 M='multi_nand3'

xnand2_b8 vdd 0 0 0 vin5 g clk gg DNAND3 M='multi_nand3'

xinv4 vin6 vout vdd 0 INV M='multi_inv4'

cload vout 0 12.60438fF

* SRAM Chain

xsram0 vdd 0 vout vbit0 vbitbar0 Sram

xsram1 vdd 0 vout vbit1 vbitbar1 Sram

xsram2 vdd 0 vout vbit2 vbitbar2 Sram

xsram3 vdd 0 vout vbit3 vbitbar3 Sram

xsram4 vdd 0 vout vbit4 vbitbar4 Sram

xsram5 vdd 0 vout vbit5 vbitbar5 Sram

xsram6 vdd 0 vout vbit6 vbitbar6 Sram

xsram7 vdd 0 vout vbit7 vbitbar7 Sram

xsram8 vdd 0 vout vbit8 vbitbar8 Sram

xsram9 vdd 0 vout vbit9 vbitbar9 Sram

xsram10 vdd 0 vout vbit10 vbitbar10 Sram

xsram11 vdd 0 vout vbit11 vbitbar11 Sram

xsram12 vdd 0 vout vbit12 vbitbar12 Sram

xsram13 vdd 0 vout vbit13 vbitbar13 Sram

xsram14 vdd 0 vout vbit14 vbitbar14 Sram       

xsram15 vdd 0 vout vbit15 vbitbar15 Sram       

xsram16 vdd 0 vout vbit16 vbitbar16 Sram    

xsram17 vdd 0 vout vbit17 vbitbar17 Sram                                    

xsram18 vdd 0 vout vbit18 vbitbar18 Sram

xsram19 vdd 0 vout vbit19 vbitbar19 Sram

xsram20 vdd 0 vout vbit20 vbitbar20 Sram

xsram21 vdd 0 vout vbit21 vbitbar21 Sram

xsram22 vdd 0 vout vbit22 vbitbar22 Sram

xsram23 vdd 0 vout vbit23 vbitbar23 Sram

xsram24 vdd 0 vout vbit24 vbitbar24 Sram

xsram25 vdd 0 vout vbit25 vbitbar25 Sram

xsram26 vdd 0 vout vbit26 vbitbar26 Sram       

xsram27 vdd 0 vout vbit27 vbitbar27 Sram       

xsram28 vdd 0 vout vbit28 vbitbar28 Sram    

xsram29 vdd 0 vout vbit29 vbitbar29 Sram                                    

xsram30 vdd 0 vout vbit30 vbitbar30 Sram

xsram31 vdd 0 vout vbit31 vbitbar31 Sram

cout vout 0 13f

*****************************************

* Analysis

*****************************************

*nomod= no model info from library

.options post=2 nomod

*.op makes hspice determines DC operating point

.op

.tran 0.1ns 16ns *sweep multi_nand3 

.meas tran tplh trig V(vin1) val='(2.5*.5)' rise=1 targ V(vout)

+ val='(2.5*.5)' rise=1

.end

Awaves

6-Stage not-not-nand-not-nand-not circuit Transcient Plot
 SHAPE  \* MERGEFORMAT 



Conclusion. Dynamic is not suitable for the decoder we need.

[image: image7.emf] 

Q  



































































































































































































































University of California, Berkeley
2
© 2006

[image: image10.wmf][image: image11.wmf]