EE145M

Lab 9 A/D Conversion and Periodic Sampling

Chih-Chieh Wang

17654794

Lab Time: Wednesday 10-1pm

Lab Station: 13

Lab Partner: Bill Hung

Aim

To interface with an external A/D converter, to use LabVIEW VI to implement handshaking signals, to explore the properties of the A/D converter, and to sample a sine wave.

1. Setup

Figure 1 Circuit Diagram for lab 9

Connect the ADC0820CCN output pins as the diagram above, and connect DGND from the FPGA board to the breadboard as well. In addition, connect power supply to Vcc, and use the +5V from the FPGA board to supply the Vref+ pin. Finally, connect the other output from the power supply to the Vin pin. The digital output will be displayed on the front panel of ADC.vi.

2. Procedure and Data Summary

2.0 Procedure

We need to follow the handshaking in order to sample the ADC and display the output on the screen. Therefore, we need a sequence structure in a loop. The handshaking steps are:

1. Set RD-bar to LOW

2. Wait for INT-bar to go LOW

3. Read the output of the PORT0 and display it in a control on your Front Panel

They can be recognized by the first 3 frames of the sequence. We wait for 10us (it could be shorter since it takes only 1.5us to convert), and then bring RD-bar HIGH for the next conversion. Repeat the steps until user presses stop.

2.1 Comparison between measured transition voltages and the linear model. Summarize your results in a table.

The linear model is give by:

Vlin(n, n+1) = V0,1 + n * ∆V, where ∆V = [V126,127 – V0,1]/126

∆V is the average spacing. In our case, it is 0.01962V

Table 2.1 Comparing measurements and the linear model

| n, transition step from n-1 to n | Vn,n+1, measured transition voltage (V) | V_lin(n, n+1), linear model (V) | Difference in V Vn,n+1 - V_lin(n,n+1) | Difference in mV Vn,n+1 - V_lin(n, n+1) | Difference in LSB (Vn,n+1 - V_lin(n, n+1))/delta_V |
| --- | --- | --- | --- | --- | --- |
| 0 | 0.01100 | 0.01100 | 0.00000 | 0.00000 | 0.00000 |
| 1 | 0.02800 | 0.03062 | -0.00262 | -2.61905 | -0.13350 |
| 2 | 0.04800 | 0.05024 | -0.00224 | -2.23810 | -0.11408 |
| 3 | 0.06700 | 0.06986 | -0.00286 | -2.85714 | -0.14563 |
| 24 | 0.47900 | 0.48186 | -0.00286 | -2.85714 | -0.14563 |
| 25 | 0.49800 | 0.50148 | -0.00348 | -3.47619 | -0.17718 |
| 49 | 0.97300 | 0.97233 | 0.00067 | 0.66667 | 0.03398 |
| 50 | 0.99400 | 0.99195 | 0.00205 | 2.04762 | 0.10437 |
| 74 | 1.46400 | 1.46281 | 0.00119 | 1.19048 | 0.06068 |
| 75 | 1.48300 | 1.48243 | 0.00057 | 0.57143 | 0.02913 |
| 99 | 1.95500 | 1.95329 | 0.00171 | 1.71429 | 0.08738 |
| 100 | 1.97600 | 1.97290 | 0.00310 | 3.09524 | 0.15777 |
| 125 | 2.46000 | 2.46338 | -0.00338 | -3.38095 | -0.17233 |
| 126 | 2.48300 | 2.48300 | 0.00000 | 0.00000 | 0.00000 |
|  |  | average | -0.00058 | -0.58163 | -0.02965 |

2.2 Differential linearity. Tabulate Vn,n+1 – Vn-1,n from your data and compare with the average step size ∆V to determine the differential linearity error

Table 2.2 Differential linearity

| N | Vn,n+1 – Vn-1,n (V) | (Vn,n+1 – Vn-1,n) /delta_V (LSB) |
| --- | --- | --- |
| 1 | 0.01700 | 0.86650 |
| 2 | 0.02000 | 1.01942 |
| 3 | 0.01900 | 0.96845 |
| 25 | 0.01900 | 0.96845 |
| 50 | 0.02100 | 1.07039 |
| 75 | 0.01900 | 0.96845 |
| 100 | 0.02100 | 1.07039 |
| 126 | 0.02300 | 1.17233 |
| Average | 0.01988 | 1.01305 |

Differential Linearity is the difference between the spacing of neighboring transition and their average spacing [Derenzo, 163]. According to Table 2.2, the average differential linearity is 1.01305, which means that the spacing of the neighboring transition and the average spacing, ∆V, is about the same. Therefore, we have good differential linearity.

2.3 RMS deviation. Compute the rms deviations between Vn,n+1 and Vlin(n,n+1):

Vrms=1Mi=1M[Vlin(ni,ni+1)−Vni,ni+1]2

where the summation is carried out only over your M measured values of Vn,n+1

Table 2.3 Vrms Calculation

| N | Vn,n+1 – Vn-1,n (V) | (Vn,n+1 – Vn-1,n)^2 |
| --- | --- | --- |
| 1 | 0.017 | 0.000289 |
| 2 | 0.02 | 0.0004 |
| 3 | 0.019 | 0.000361 |
| 25 | 0.419 | 0.175561 |
| 50 | 0.021 | 0.000441 |
| 75 | 0.019 | 0.000361 |
| 100 | 0.021 | 0.000441 |
| 126 | 0.023 | 0.000529 |

Vrms = 0.149325V

2.4 Sampling frequency

Sampling frequency = 1 / 20us = 50000 samples/sec

2.5 Power-supply rejection ratio. For the V0,1 and V126,127 transition voltages, compute the power-supply rejection ration as:

∆Vn,n+1/∆VS

where ∆Vn,n+1 is the change in the transition voltage Vn,n+1 for a change in the power supply voltage ∆VS.

Table 2.4 Power-supply rejection ratio

| n | Vn,n+1 measured at 5V supply (V) | Vn,n+1 measured at 4.85V supply (V) | power rejection ratio |
| --- | --- | --- | --- |
| 0 | 0.011 | 0.006 | 0.033 |
| 126 | 2.483 | 2.514 | -0.207 |
|  |  | average | 0.1200 |

Note: The power rejection ratio should be quite similar at different voltages, but we didn’t observe it in the experiment. We should take more data to do the comparison.

3. Discussion and conclusions

3.1 Discuss the timing diagram you measured in procedure section 3.3.

RD|

INT|

100us

1.48us

10us

10us

Figure 2 handshaking waveforms

78.52us

The purpose of handshaking is check. First, the microcomputer sets RD| to LOW to signal the external A/D converter that the microcomputer is ready for data. The A/D converter will see RD| being LOW, and will convert analog input to digital output. Once the conversion is done, the A/D converter will bring INT| LOW which signals the microcomputer that data is available. According to the Figure 2, it takes about 1.48us to do the conversion, which is very close to 1.5us as stated in the lab handout. RD| and INT| are LOW in the next 10us due to the first Wait in the fourth sequence structure. Next, we bring RD| high to prepare for the next conversion, and Wait for another 10us. Since the loop timer is set to 100us, we need to wait until it reaches 100us. In fact, according to the waveforms in Figure 2, the whole conversion cycle could shorten to around 10us.

3.2 Discuss how well you were able to measure the transition voltages in procedure section 3.4.

Transition voltages were not easy to measure because the transition was not sharp enough. As the analog input voltage changed slightly, the digital output fluctuated. It was hard to determine where the exactly transition voltage was.

3.3 Discuss any hysteresis you observed in procedure section 3.5 and possible causes.

According to the experiment, we found that transition voltages for decreasing input are different from transition voltages for increasing input, which implies that ADC0820 A/D converter has hysteresis, a property of systems whose states depend on their immediate history. Hysteresis usually exists in the comparator circuit (see Figure 3.9 extracted from the book). The positive feedback of comparator makes the circuit more stable. Because of hysteresis, there are two different switching thresholds so that a HIGH or a LOW output value will not change accidentally if there is some noise in the input (see Figure 3.10 extracted from the book).

Recall ADC0820 A/D converter uses subranging flash method, which is a hybrid between the successive approximation and the flash converters [Derenzo, 171]. Examining the circuits for successive approximation and the flash converters (see Figure 3.17 extracted from the book), successive-approximation uses a comparator. Therefore, we observed different transition voltages (see Table 3.3).

Table 3.3 Transition voltages for increasing inputs and decreasing inputs

| V1,2 | 0.028V |
| --- | --- |
| V0,1 | 0.011V |
| V2,1 | 0.031V |
| V1,0 | 0.015V |

3.4 Discuss the importance of good power-supply rejection for A/D converters used in battery-operated equipment.

The smaller the power-supply rejection ratio, the better the A/D converter is. Good power-supply rejection can produce more accurate measurements even when the power supply is not stable. Good power-supply rejection ratio is essential to battery-operated equipment since batteries can’t produce constant voltage in reality. If we have a good power-supply rejection A/D converter, as long as we keep the reference voltage constant, we would expect the output to be the same even though the batteries might undergoes a voltage drop.

3.5 Discuss procedure section 4. Consider how these techniques can be used to sample analog waveforms and their limitations.

In procedure section 4, the second frame of the sequence structure does the similar job to the first part of the lab. The only difference is that we store the digital output to memory on the FPGA. After writing all the data to memory, the FPGA signals the microcomputer. The Lab9Host.vi on the microcomputer will handle the interrupt. One of the limitations is that we need to sample analog waveforms and save them on the FPGA and then download them on to windows to process them. Another limitation is speed since clock rate and handshaking can be very tricky with 2 circuits with different clocks. Also, if the power supply to one side of the circuit suddenly turns off, we would not get the correct samples.

3.6 Describe how your A/D differed from the specifications promised in the data sheets in terms of resolution, absolute (unadjusted) accuracy, linearity (relative accuracy), differential linearity, conversion speed, and power-supply rejection ratio.

For absolute accuracy, the ideal transition voltages are described by

V(n,n+1)=Vref−+(n+12)ΔV where ΔV=(Vref+−Vref−2N−1) and Vref+=5V,Vref−=0V

∆V = 0.03937V

Table 3.6-1 Absolute accuracy

| n, transition step from n-1 to n | Vn,n+1, measured transition voltage (V) | V(n, n+1), ideal model (V) | Difference in V Vn,n+1 - V(n, n+1) | Difference in LSB (Vn,n+1 - V(n, n+1))/delta_V |
| --- | --- | --- | --- | --- |
| 0 | 0.01100 | 0.00000 | 0.01100 | 0.56068 |
| 1 | 0.02800 | 0.01969 | 0.00832 | 0.42382 |
| 2 | 0.04800 | 0.03937 | 0.00863 | 0.43988 |
| 3 | 0.06700 | 0.05906 | 0.00795 | 0.40496 |
| 24 | 0.47900 | 0.47244 | 0.00656 | 0.33437 |
| 25 | 0.49800 | 0.49213 | 0.00587 | 0.29945 |
| 49 | 0.97300 | 0.96457 | 0.00843 | 0.42994 |
| 50 | 0.99400 | 0.98425 | 0.00975 | 0.49697 |
| 74 | 1.46400 | 1.45669 | 0.00731 | 0.37260 |
| 75 | 1.48300 | 1.47638 | 0.00663 | 0.33768 |
| 99 | 1.95500 | 1.94882 | 0.00618 | 0.31525 |
| 100 | 1.97600 | 1.96850 | 0.00750 | 0.38228 |
| 125 | 2.46000 | 2.46063 | -0.00063 | -0.03186 |
| 126 | 2.48300 | 2.48031 | 0.00269 | 0.13711 |
|  |  | Average | 0.00687 | 0.35022 |

Table 3.6-2 Comparisons between measured and Datasheet values

|  | Measured | Datasheet |
| --- | --- | --- |
| Resolution | 7-bit | 8-bit |
| Absolute (unadjusted) accuracy | 0.35022 LSB | +/- 1 LSB |
| Linearity (relative accuracy) | -0.02965 LSB | N/A |
| Differential linearity | 1.01305 LSB | N/A |
| Conversion speed | 1.48us | 1.5us max |
| Power-supply rejection ratio | 0.1200 | +/- 1/16 |

Note 1: According to the TA, Peter Lau, we were following the original textbook lab, which only connects 7 of the 8 A/D lines, so the resolution was 7-bit.

Note 2: Linearity is taken from Table 2.1

Note 3: The datasheet is missing the information about linearity and differential linearity

We have very good absolute accuracy, linearity, and differential linearity. Comparing to the specifications in the datasheet, the conversion rate measured is very close to the datasheet. However, the power-supply rejection ratio is larger than the one in the datasheet. It is possible that we didn’t have enough data so that the two figures do not quite agree since power rejection ratio shouldn’t change much even if the input changes.

Questions

4.1 Was the V0,1 transition voltage what you expected? Was the V126,127 transition voltage what you expected?

I would expect that V0,1 is 1LSB with 5V reference voltage = 1 * 5V/255 = 0.0196V, and V126,127 = 127*1LSB with 5V supply = 2.489V. It turned out that V0,1 was 0.011V, and V126,127 was 2.483V. The results are very accurate.

4.2 What was the largest millivolt deviation between your transition data and the linear model?

According to Table 2.1, transition voltage V125,126 differed the most from the linear model, the deviation is 3.38095mV.

4.3 What was your sampling frequency in procedure section 4? What is the maximum frequency sine wave that your system can sample without aliasing?

From Data Summary 2.4, the sampling frequency is 50000 samples/sec. Therefore, in order to avoid aliasing, the maximum frequency sine wave that the system can sample is half of the sampling frequency, 25000 Hz, according to the Nyquist Theorem

4.4 In characterizing an A/D converter, why is it better to measure transition voltages rather than recording digital output values for a number of random analog input voltages?

Because the output of the A/D converter is discrete, we would like to know the range of analog input for each discrete digital output. Otherwise, measuring random analog input voltages is meaning less, and it will not help us to find the linear relationship between the analog input and the digital output.

4.5 How would you use a high-resolution D/A converter and a computer with a digital I/O port to automate the measurement of A/D transition voltages?

Assume that A/D and D/A follow their linear models. Connect the digital output port of the PC to the input of D/A converter, and then connect the output of D/A converter to the input of the A/D converter. Finally, connect the output of A/D converter to the digital input of the PC. Next, write a program to control the digital input of the D/A. Since it follows the linear model, we can predict the analog output of D/A. Set up 2 arrays; one is used to record the digital output of the A/D when there is a transition; the other one is used to record the predicted analog output of the D/A when there is a transition. Increment the digital input of the D/A, and observe any changes in the digital output of the A/D. If there is a change, record the digital output of the A/D and the predicted analog output of D/A to the arrays. Keep incrementing the digital input of the D/A until it reaches the maximum value. Thus, this program can automatically measure the A/D transition voltages.

5. Program and laboratory data sheets

5.1 Include printouts of your VI front panel and block diagram.

Figure 5.1-1: The Front Panel of ADC.vi

Figure 5.1-2: The Block Diagram of ADC.vi

6. Extra Credit: Sampling a sine wave

6.1. Procedure:

We need two VI’s, Lab9Host.vi and Lab9FPGA.vi, to sample a sine wave. Lab9Host.vi is executed on the PC while Lab9FPGA.vi is downloaded and executed on the FPGA. The two VI’s are connected by interrupt request (IRQ). The goal of Lab9FPGA.vi is to convert the analog inputs to digital outputs like the first part of the lab until it collects the number of points indicated on the front panel of Lab9Host.vi. Examining the original Lab9FPGA.vi template, the second frame of the sequence structure showed the FPGA would write the received data to memory available on the FPGA. Therefore, we need to change it so that the FPGA can interface with A/D converter and store the digital outputs of the A/D converter. Next, the FPGA will signal the PC, Lab9Host.vi will handle this interrupt; it will read the memory on FPGA, and put the data into a text file.

Figure 6.1-1 The Front Panel of Lab9Host.vi

Figure 6.1-2 The Block Diagram of Lab9Host.vi

Figure 6.1-3 The Front Panel Lab9FPGA.vi

Figure 6.1-4 the original Lab9FPGA.vi template

Figure 6.1-5 the modified Lab9FPGA.vi block diagram

6.2 Data Summary

| Lateness |  |
| --- | --- |
| Setup |  |
| Procedure and Data Summary |  |
| Analysis |  |
| Discussion |  |
| Questions |  |
| Clarity |  |
| Total grade |  |